TABLE OF CONTENTS

- Introduction………………………………………………………………………………………………………………………..
    1.1 Background of the Project……………………………………………………………………………………………..
    1.2 Problem Definition…………………………………………………………………………………………………………
    1.3 Objectives of the Project………………………………………………………………………………………………..
    1.4 Scope of the Project……………………………………………………………………………………………………….

- Literature Review………………………………………………………………………………………………………………..
    2.1 Overview of Existing Solutions……………………………………………………………………………………….
    2.2 Limitations of the Current System………………………………………………………………………………….
    2.3 Research on References Studied…………………………………………………………………………………….

- Proposed Software Solution………………………………………………………………………………………………..
    3.1 Concept of Proposed Solution………………………………………………………………………………………..
    3.2 Advantages over Existing System……………………………………………………………………………………
    3.3 System Features…………………………………………………………………………………………………………….

- Development Environment…………………………………………………………………………………………………
    4.1 Hardware Environment………………………………………………………………………………………………….
    4.2 Software Environment……………………………………………………………………………………………………
        4.2.1 Operating System…………………………………………………………………………………………………….
        4.2.2 Programming Language…………………………………………………………………………………………..
        4.2.3 Development Tools / IDE………………………………………………………………………………………...
        4.2.4 Libraries and Frameworks……………………………………………………………………………………….

- Software Design………………………………………………………………………………………………………………….
    5.1 Software Architecture……………………………………………………………………………………………………
    5.2 Block Diagram……………………………………………………………………………………………………………….
    5.3 Flow Chart…………………………………………………………………………………………………………………….
    5.4 Module Description………………………………………………………………………………………………………
        5.4.1 User Authentication Module……………………………………………………………………………………
        5.4.2 Auto Mode Simulation Module……………………………………………………………………………….
        5.4.3 Manual Mode Simulation Module……………………………………………………………………………
        5.4.4 PDW Interleaving Module……………………………………………………………………………………….
        5.4.5 De-Interleaving Module………………………………………………………………………………………….
        5.4.6 Emitter Tracking and Analysis Module……………………………………………………………………..
        5.4.7 Data Storage and Export Module…………………………………………………………………………….
    5.5 Data Flow Diagram…………………………………………………………………………………………………………
        5.5.1 Level-0 Data Flow Diagram………………………………………………………………………………………
        5.5.2 Level-1 Data Flow Diagram………………………………………………………………………………………
        5.5.3 Data Stores Used……………………………………………………………………………………………………..

- Implementation………………………………………………………………………………………………………………….
    6.1 Step-by-Step Working of the System………………………………………………………………………………
    6.2 Algorithms Used…………………………………………………………………………………………………………….
        6.2.1 Density-Based Spatial Clustering of Applications with Noise (DBSCAN)……………………
        6.2.2 Hierarchical Density-Based Spatial Clustering (HDBSCAN)……………………………………….
        6.2.3 Feature Scaling and Normalization………………………………………………………………………….
    6.3 Pseudocode……………………………………………………………………………………………………………………
        6.3.1 Pseudocode for PDW Simulation Module………………………………………………………………..
        6.3.2 Pseudocode for Feature Selection Module……………………………………………………………..
        6.3.3 Pseudocode for De-Interleaving using DBSCAN……………………………………………………….
        6.3.4 Pseudocode for De-Interleaving using HDBSCAN……………………………………………………..
        6.3.5 Pseudocode for Emitter Tracking…………………………………………………………………………….

- Results………………………………………………………………………………………………………………………………..
    7.1 Output Screenshots……………………………………………………………………………………………………….
        7.1.1 Auto Mode – PDW Simulation Output…………………………………………………………………….
        7.1.2 Manual Mode – Configured Emitter Simulation Output………………………………………….
        7.1.3 Interleaved PDW Data View…………………………………………………………………………………….
        7.1.4 De-Interleaved Emitter Summary…………………………………………………………………………….
        7.1.5 Emitter Tracking Output………………………………………………………………………………………….
        7.1.6 De-Interleaving Summary Statistics…………………………………………………………………………

7.2 Observations……………………………………………………………………………………………………………………
        7.2.1 Effectiveness of PDW Simulation……………………………………………………………………………..
        7.2.2 Impact of Feature Selection on Clustering………………………………………………………………
        7.2.3 DBSCAN Performance Characteristics……………………………………………………………………..
        7.2.4 HDBSCAN Performance Characteristics…………………………………………………………………..
        7.2.5 Interpretation of Unassigned Pulses………………………………………………………………………
        7.2.6 Validation Against Ground Truth……………………………………………………………………………..
        7.2.7 Overall System Behavior………………………………………………………………………………………….

7.3 Performance Analysis………………………………………………………………………………………………………
        7.3.1 De-Interleaving Accuracy………………………………………………………………………………………..
        7.3.2 Impact of Emitter Density………………………………………………………………………………………
        7.3.3 Feature Dimensionality Analysis…………………………………………………………………………….
        7.3.4 Algorithm Scalability………………………………………………………………………………………………
        7.3.5 Robustness to Parameter Variations……………………………………………………………………..
        7.3.6 DBSCAN vs HDBSCAN Comparison…………………………………………………………………………..
        7.3.7 Real-Time Applicability……………………………………………………………………………………………

- Testing………………………………………………………………………………………………………………………………..
    8.1 Test Cases………………………………………………………………………………………………………………………
    8.2 Test Results……………………………………………………………………………………………………………………

- Conclusion………………………………………………………………………………………………………………………..
    9.1 Summary of Work Done………………………………………………………………………………………………
    9.2 Achievements………………………………………………………………………………………………………………
    9.3 Learning Outcomes……………………………………………………………………………………………………..

- Future Scope………………………………………………………………………………………………………………………
    10.1 Possible Enhancements……………………………………………………………………………………………….

References……………………………………………………………………………………………………………………………….

- INTRODUCTION

1.1 Background of the Project

Radar Electronic Support Measures (ESM) systems are passive surveillance systems used to intercept and analyze radar signals in the electromagnetic spectrum. These systems collect radar emissions from multiple sources and convert them into structured data known as Pulse Descriptor Words (PDWs), which contain key parameters such as time of arrival, frequency, pulse width, pulse repetition interval, amplitude, and direction of arrival. PDWs form the fundamental input for radar signal analysis and emitter identification in ESM applications.

In real operational environments, multiple radar emitters operate simultaneously, causing their pulses to arrive in an interleaved manner at the receiver. Each emitter may follow different operational modes such as fixed frequency, frequency agile operation, fixed PRI, staggered PRI, or jittered PRI. Due to this interleaving, the received PDW stream does not directly indicate which pulses belong to a particular emitter, making accurate analysis and threat assessment difficult without additional processing.

PDW de-interleaving is therefore a critical function in ESM systems, where the objective is to segregate interleaved PDWs into distinct groups corresponding to individual radar emitters. To study and validate this process effectively, realistic simulation of PDWs and robust de-interleaving techniques are required. This project focuses on developing a real-time PDW simulation and de-interleaving framework that models practical radar behaviors and enables accurate separation of emitter signals using data-driven approaches.

1.2 Problem Definition

Radar Electronic Support Measures (ESM) systems are passive surveillance systems used to intercept and analyze radar signals in the electromagnetic spectrum. These systems collect radar emissions from multiple sources and convert them into structured data known as Pulse Descriptor Words (PDWs), which contain key parameters such as time of arrival, frequency, pulse width, pulse repetition interval, amplitude, and direction of arrival. PDWs form the fundamental input for radar signal analysis and emitter identification in ESM applications.

In real operational environments, multiple radar emitters operate simultaneously, causing their pulses to arrive in an interleaved manner at the receiver. Each emitter may follow different operational modes such as fixed frequency, frequency agile operation, fixed PRI, staggered PRI, or jittered PRI. Due to this interleaving, the received PDW stream does not directly indicate which pulses belong to a particular emitter, making accurate analysis and threat assessment difficult without additional processing.

PDW de-interleaving is therefore a critical function in ESM systems, where the objective is to segregate interleaved PDWs into distinct groups corresponding to individual radar emitters. To study and validate this process effectively, realistic simulation of PDWs and robust de-interleaving techniques are required. This project focuses on developing a real-time PDW simulation and de-interleaving framework that models practical radar behaviors and enables accurate separation of emitter signals using data-driven approaches.

1.3 Objectives of the Project

The objectives of the project are as follows:

- To design and develop a realistic PDW simulation framework that generates continuous-time Pulse Descriptor Word streams representing multiple radar emitters operating simultaneously in an Electronic Support Measures (ESM) environment.

- To simulate different radar emitter characteristics, including fixed frequency, frequency agile, fixed PRI, staggered PRI, and jittered PRI modes, incorporating practical parameter tolerances such as frequency, PRI, pulse width, amplitude, and direction of arrival.

- To implement an automated PDW de-interleaving mechanism using density-based clustering techniques to accurately segregate interleaved radar pulses and associate them with their respective emitters.

- To support multi-feature de-interleaving, enabling clustering based on frequency, PRI, pulse width, and direction of arrival, either individually or in combination, to evaluate algorithm performance under varied conditions.

- To provide an interactive and user-friendly software interface for configuring simulation parameters, controlling real-time PDW generation, and visualizing interleaved and de-interleaved data.

- To enable emitter-wise tracking and analysis, allowing users to inspect the characteristics and pulse patterns of individual radar emitters after de-interleaving.

- To evaluate the performance and robustness of de-interleaving algorithms under different operational scenarios, including dense emitter environments and closely spaced parameter conditions.

1.4 Scope of the Project

The scope of this project is focused on the design and implementation of a software-based system for the simulation and de-interleaving of radar Pulse Descriptor Words (PDWs) in an Electronic Support Measures (ESM) context. The system is intended to model realistic radar emitter behavior and support the analysis of interleaved pulse streams using data-driven techniques.

The project includes the simulation of multiple radar emitters operating simultaneously with different operational modes such as fixed frequency, frequency agile, fixed PRI, staggered PRI, and jittered PRI. The simulation incorporates practical parameter tolerances in frequency, pulse repetition interval, pulse width, amplitude, and direction of arrival to closely resemble real-world radar environments. The PDW data is generated in continuous time and supports real-time-like analysis.

The scope also covers the implementation of automated PDW de-interleaving using density-based clustering algorithms. The system enables multi-feature clustering, emitter-wise tracking, and visualization of interleaved and de-interleaved PDW data through an interactive user interface. However, the project does not include hardware-level signal acquisition, low-level RF signal processing, or classification of radar types based on emitter identity.

- LITERATURE REVIEW

2.1 Overview of existing solution

Radar Pulse Descriptor Word (PDW) de-interleaving has been an active area of research in Electronic Support Measures (ESM) systems due to the increasing complexity of modern radar environments. Traditional solutions primarily rely on classical signal processing techniques, where PDWs are analyzed using parameters such as Time of Arrival (TOA), Pulse Repetition Interval (PRI), Radio Frequency (RF), Pulse Width (PW), and Direction of Arrival (DOA). Techniques such as PRI histograms, autocorrelation methods, and PRI transform-based approaches have been widely used to separate pulse trains belonging to different emitters. These methods are computationally efficient and interpretable but tend to perform poorly in dense environments involving frequency-agile radars, staggered PRI patterns, jitter, and missing pulses.

To overcome these limitations, feature-based clustering approaches have been introduced. In these methods, PDWs are represented as feature vectors and grouped using unsupervised clustering algorithms such as hierarchical clustering and density-based methods. Density-based algorithms, particularly DBSCAN, gained popularity due to their ability to identify clusters of arbitrary shape and handle outliers. These approaches improve robustness against interleaving but are highly sensitive to parameter selection and require well-defined tolerances to function effectively in real-time scenarios.

Recent advancements have explored optimization-based and machine learning-driven solutions. Optimization techniques model de-interleaving as an assignment or transport problem, improving pulse-to-emitter association accuracy. Meanwhile, deep learning approaches—including recurrent networks, semantic segmentation, and transformer-based models—have demonstrated promising performance by learning complex relationships between PDWs. However, these methods often require large labeled datasets, high computational resources, and reduced interpretability, limiting their deployment in operational ESM systems.

Overall, while existing solutions provide valuable insights and partial solutions, they face challenges related to scalability, adaptability, and real-time applicability. This has motivated the development of hybrid and simulation-driven approaches that combine realistic PDW generation with robust unsupervised clustering, forming the basis for the proposed system in this project.

2.2 Limitations of the Current System

Despite significant research in radar PDW de-interleaving, existing systems exhibit several limitations when applied to modern and dense radar environments. Traditional de-interleaving approaches based on fixed thresholds, PRI histograms, and rule-based decision logic lack adaptability and often fail under conditions such as frequency agility, staggered or jittered PRI, pulse loss, and overlapping emitters. These methods assume stable radar behavior and are not well suited for dynamically changing emitter characteristics encountered in real-world Electronic Support Measures (ESM) scenarios.

Many clustering-based solutions, while more flexible, are highly dependent on manually tuned parameters such as clustering radius, minimum samples, and feature scaling. Incorrect parameter selection can lead to over-clustering, under-clustering, or misclassification of pulses. Additionally, several approaches treat noise and outliers inconsistently, resulting in inaccurate emitter counts even in the absence of true noise. This sensitivity makes such systems unreliable for operational deployment without extensive human intervention or prior knowledge of emitter behavior.

Advanced machine learning and deep learning-based techniques address some of these challenges but introduce new constraints. These methods require large labeled datasets, significant computational resources, and complex training procedures, which are often impractical in defence environments where data availability is limited and system transparency is critical. Furthermore, black-box models reduce interpretability, making it difficult for analysts to validate or trust the results during real-time intelligence assessment.

In summary, current PDW de-interleaving systems struggle to balance accuracy, adaptability, interpretability, and real-time performance. These limitations highlight the need for a robust, simulation-driven, and user-configurable solution capable of handling fixed, agile, and staggered emitters under continuous time conditions—motivating the development of the proposed system.

2.3 Research on References Studied

An extensive study of existing literature was conducted to understand current methodologies and research trends in radar PDW de-interleaving and Electronic Support Measures (ESM) systems. The reviewed works primarily focus on pulse-based feature extraction, emitter characterization, and clustering-based separation techniques using parameters such as frequency, Pulse Repetition Interval (PRI), pulse width, and Direction of Arrival (DOA).

Several studies propose classical statistical and rule-based approaches, including PRI histogram analysis, sequential difference methods, and time–frequency correlation techniques for de-interleaving radar signals. While these methods perform adequately for fixed-parameter emitters, the literature highlights their limitations when dealing with frequency-agile radars, staggered or jittered PRI patterns, and high pulse density environments. These findings emphasize the need for adaptive techniques capable of handling modern radar complexities.

More recent research explores density-based clustering algorithms such as DBSCAN and HDBSCAN for PDW de-interleaving. These algorithms are shown to be effective in separating overlapping emitters without prior knowledge of the number of radars. Studies also indicate that multi-feature clustering improves accuracy compared to single-parameter approaches. However, challenges related to parameter sensitivity, cluster stability, and noise handling are frequently reported.

Additionally, a few works investigate machine learning and deep learning-based solutions for radar signal separation. Although these approaches demonstrate high accuracy in controlled conditions, they rely heavily on labeled datasets and extensive training, which limits their applicability in real-time and operational ESM systems.

The insights gained from these references influenced the design of the proposed system by adopting a hybrid approach that combines realistic PDW simulation with density-based clustering, emphasizing adaptability, interpretability, and real-time processing. This research foundation ensures that the proposed solution aligns with established techniques while addressing the practical limitations identified in existing studies.

- PROPOSED SOFTWARE SOLUTION

3.1 Concept of Proposed Solution

The proposed software solution aims to design and implement a standalone, offline Electronic Support Measures (ESM) application capable of simulating radar Pulse Descriptor Words (PDWs) and performing accurate de-interleaving of interleaved radar signals in a real-time operational context. The system is developed to replicate realistic radar environments and support the analysis of modern radar emitters exhibiting fixed, frequency-agile, and staggered PRI behaviors.

The core concept of the proposed solution is based on a two-phase architecture: PDW Simulation Phase and PDW De-Interleaving Phase. In the simulation phase, radar emitters are modeled with configurable parameters such as frequency, PRI, pulse width, amplitude, and direction of arrival, along with realistic tolerances to emulate real-world measurement variations. The system supports both automated and manual configuration modes, allowing flexibility in generating diverse radar scenarios. PDWs are generated in a continuous time framework with periodic updates, closely mimicking real-time ESM data acquisition.

In the de-interleaving phase, the generated interleaved PDWs are processed using density-based clustering techniques, primarily DBSCAN and HDBSCAN. These algorithms group pulses belonging to the same emitter based on selected signal features without requiring prior knowledge of the number of emitters. Feature selection and adaptive parameter tuning are incorporated to enhance clustering accuracy across different operational scenarios. The output is presented through a structured three-pane interface, displaying raw interleaved data, de-interleaved emitter clusters, and detailed pulse-level tracking for each detected emitter.

Overall, the proposed solution emphasizes realism, adaptability, and interpretability, making it suitable for ESM analysis, algorithm evaluation, and training purposes. By integrating realistic PDW generation with robust de-interleaving techniques in a standalone application, the system provides an effective and practical platform for studying radar signal separation in complex electromagnetic environments.

3.2 Advantages over existing system

The proposed software solution offers several significant advantages over existing PDW de-interleaving approaches and tools, particularly in terms of realism, flexibility, and usability within an Electronic Support Measures (ESM) environment.

One of the key advantages is the integration of realistic PDW simulation with de-interleaving in a single standalone application. Many existing systems rely on pre-recorded datasets or proprietary radar data, limiting experimentation and repeatability. In contrast, the proposed system generates PDWs dynamically using configurable emitter parameters and realistic tolerances, enabling controlled testing under diverse radar scenarios without dependency on external datasets.

Another major advantage is the support for multiple emitter behaviors, including fixed frequency, frequency-agile, jittered PRI, and staggered PRI emitters. Traditional approaches often focus on limited radar models or assume ideal signal conditions. The proposed solution accommodates modern radar characteristics and mixed-emitter environments, improving its applicability to real-world ESM operations.

The system also provides adaptive, density-based de-interleaving using DBSCAN and HDBSCAN algorithms, which do not require prior knowledge of the number of emitters. This is a significant improvement over conventional clustering techniques that depend on predefined cluster counts. Automatic parameter tuning and feature selection further enhance clustering robustness across varying signal conditions.

From a usability perspective, the application is fully offline and standalone, making it suitable for secure environments where internet access and external dependencies are restricted. The intuitive graphical interface, combined with a structured three-window visualization of interleaved data, de-interleaved results, and emitter-level tracking, improves interpretability and user understanding of algorithm performance.

Overall, the proposed system delivers a more flexible, realistic, and analyst-friendly solution compared to existing systems, making it well-suited for research, evaluation, and training applications in radar ESM signal analysis.

3.3 System Features

The proposed PDW de-interleaving system incorporates a comprehensive set of features designed to support realistic simulation, robust analysis, and effective visualization of radar emitter behavior in an ESM environment.

- Standalone and Offline Operation
The application operates completely offline as a standalone system, making it suitable for secure and restricted environments where network access is not permitted.

- User Authentication and Access Control
A secure login and signup mechanism ensures controlled access to the system, supporting multi-user operation and role-based usage.

- Real-Time PDW Simulation
The system simulates PDW data in a continuous time manner with updates every fixed time window, closely resembling real-time radar signal reception.

- Auto Mode Simulation
Automatically generates interleaved PDWs using configurable parameter ranges and emitter distributions, enabling rapid testing across diverse radar scenarios.

- Manual Mode Simulation
Allows detailed, emitter-wise configuration of radar parameters, including frequency types (fixed/agile), PRI types (fixed/jittered/staggered), pulse width, amplitude, and DOA, providing fine-grained control over simulation scenarios.

- Support for Modern Radar Behaviors
Simulates fixed frequency, frequency-agile, jittered PRI, and staggered PRI emitters with realistic tolerances to reflect practical radar operating conditions.

- Configurable Parameter Tolerances
Enables users to adjust tolerance values for frequency, PRI, pulse width, DOA, and amplitude, facilitating experimentation under varying signal uncertainty levels.

- Live PDW Buffering and Data Management
Maintains continuously updated interleaved PDW streams with automatic sorting and structured storage for subsequent analysis.

- Density-Based De-Interleaving Algorithms
Implements DBSCAN and HDBSCAN algorithms for PDW de-interleaving without requiring prior knowledge of the number of emitters.

- Adaptive Algorithm Parameter Selection
Automatically tunes clustering parameters to improve de-interleaving accuracy across different datasets and feature combinations.

- Feature-Based De-Interleaving
Supports de-interleaving based on frequency, PRI, pulse width, and DOA, individually or in combination.

- Three-Window Analytical Visualization
Displays interleaved PDWs, de-interleaved emitter summaries, and detailed emitter-level tracking in a clear and intuitive multi-window layout.

- Exportable Results
Allows saving of interleaved and de-interleaved PDW datasets for further analysis or reporting.

- DEVELOPMENT ENVIRONMENT

4.1 Hardware Environment

The development and testing of the proposed Radar PDW Simulation and De-Interleaving System were carried out on a standard computing platform that is commonly available in laboratory and academic environments. The hardware configuration was selected to ensure smooth execution of real-time PDW simulation, clustering algorithms, and interactive visualization without requiring specialized or high-end hardware.

The system was implemented and validated on a personal computer with the following representative hardware specifications:

- Processor: Intel® Core™ i5 / equivalent multi-core processor

- Clock Speed: ≥ 2.4 GHz

- RAM: 8 GB (minimum)

- Storage: 256 GB SSD / HDD

- Architecture: 64-bit system

- Display: Standard monitor supporting graphical visualization

The selected hardware configuration is sufficient to support continuous PDW generation, real-time de-interleaving using density-based clustering algorithms, and simultaneous rendering of multiple data views within the user interface. The system does not rely on any specialized signal processing hardware, making it portable and easily deployable across different workstations.

This hardware setup ensures that the proposed solution can be executed efficiently in both development and demonstration environments, including research laboratories and training facilities.

4.2 Software Environment

The proposed Radar PDW Simulation and De-Interleaving System was developed using a stable and widely adopted software environment to ensure reliability, scalability, and ease of maintenance. The selected software tools and technologies support real-time data processing, interactive visualization, and implementation of advanced clustering algorithms.

The software environment used for the development of this system is described below.

4.2.1 Operating System

The application was developed and tested on the following operating system:

- Operating System: Microsoft Windows 10 / Windows 11 (64-bit)

The Windows operating system provides a stable development platform with extensive support for Python-based applications, data science libraries, and graphical user interface frameworks. It also ensures compatibility with commonly used development tools and deployment environments.

4.2.2 Programming Language

- Programming Language: Python (Version 3.x)

Python was chosen as the primary programming language due to its simplicity, readability, and strong support for scientific computing and data analysis. Python enables rapid prototyping and efficient implementation of complex algorithms such as DBSCAN and HDBSCAN, which are essential for PDW de-interleaving. Its extensive ecosystem of libraries also supports real-time simulation and interactive user interfaces.

4.2.3 Development Tools / IDE

- Integrated Development Environment (IDE): Visual Studio Code (VS Code)

Visual Studio Code was used for code development and debugging due to its lightweight nature, robust Python support, and integrated debugging capabilities. It facilitates efficient code management, testing, and version control during the development process.

4.2.4 Libraries and Frameworks

The following Python libraries and frameworks were used in the implementation of the system:

- Streamlit: Used for developing an interactive web-based user interface for simulation control and result visualization.

- NumPy: Used for numerical computations and random data generation for PDW simulation.

- Pandas: Used for structured data handling, storage, and manipulation of PDW datasets.

- Scikit-learn: Used for implementing density-based clustering algorithms such as DBSCAN and HDBSCAN.

- Matplotlib: Used for basic data visualization and plotting during analysis and debugging.

These libraries provide efficient tools for real-time PDW generation, clustering-based de-interleaving, and structured visualization of results, enabling the development of a robust and modular system.

- SOFTWARE DESIGN

5.1 Software Architecture

The proposed PDW Simulation and De-Interleaving System follows a modular, layered software architecture designed to support real-time data generation, processing, and analysis. The architecture ensures separation of concerns, allowing independent development and testing of simulation, de-interleaving, and visualization components. This design improves scalability, maintainability, and adaptability of the system for future enhancements.

At a high level, the system architecture consists of four major layers: the User Interface Layer, Simulation Layer, Processing (De-Interleaving) Layer, and Data Management Layer. Each layer performs a specific role and communicates with other layers through well-defined data flows. This layered approach closely resembles operational Electronic Support Measure (ESM) processing chains used in real-world radar environments.

The architecture supports continuous-time PDW generation, where pulse data is produced in fixed time windows and stored in memory buffers. These interleaved PDWs are then passed to the de-interleaving module, which applies density-based clustering algorithms to segregate pulses belonging to individual emitters. The results are displayed through an interactive user interface, enabling users to analyze interleaved data, de-interleaved emitters, and tracked emitter parameters in real time.

Figure 5.1: System Architecture of Radar PDW De-Interleaving Framework

Architectural Layers Description

1. User Interface Layer

This layer provides an interactive interface for users to configure simulation parameters, control the execution of the system, and visualize results. It includes:

- Login and authentication interface

- Auto Mode and Manual Mode configuration panels

- Feature selection for de-interleaving

- Tabular visualization of interleaved and de-interleaved PDW data

The UI layer ensures ease of use and allows users to understand the system behavior without requiring in-depth knowledge of the underlying algorithms.

2. Simulation Layer

The simulation layer is responsible for generating realistic PDW data streams. It simulates different types of radar emitters, including:

- Fixed frequency emitters

- Frequency agile emitters

- Staggered PRI emitters

This layer generates PDWs in continuous time windows using configurable tolerances, closely emulating real-world radar pulse behavior.

3. Processing (De-Interleaving) Layer

This layer performs the core functionality of the system—PDW de-interleaving. It processes interleaved PDWs using density-based clustering algorithms such as:

- DBSCAN

- HDBSCAN

Feature normalization and automatic parameter tuning are applied to improve clustering accuracy. The output of this layer is a set of de-interleaved emitters with associated pulse data.

4. Data Management Layer

The data management layer handles storage and organization of PDW data and results. It maintains:

- In-memory buffers for continuous PDW streams

- CSV-based persistent storage for interleaved and de-interleaved datasets

- Structured data formats for analysis and reporting

This layer ensures reliable data handling and traceability throughout the system lifecycle.

5.2 Block Diagram

The block diagram represents the overall working of the proposed Real-Time PDW Simulation and De-Interleaving System. It illustrates the flow of data from user input through simulation, processing, and final de-interleaved output.

The system begins with the user interface, where the user selects the mode of operation (Auto or Manual), configures emitter parameters, and sets tolerance values. Based on this configuration, the PDW Simulation Engine generates pulse descriptor words in a continuous-time manner, simulating realistic radar emitter behavior including fixed, frequency-agile, and staggered PRI emitters.

The generated PDWs are interleaved to form a composite pulse stream, which is then passed to the de-interleaving module. Feature selection and normalization are applied before performing density-based clustering using DBSCAN or HDBSCAN algorithms. The clustering results are used to identify individual emitters, and the final output presents de-interleaved PDWs along with emitter-wise tracking information.

This block diagram provides a clear high-level understanding of the system workflow and highlights the modular design adopted in the proposed solution.

Figure 5.2: Block Diagram of Radar PDW De-Interleaving System

5.3 Flow Chart

The system operation begins with user authentication, ensuring secure access to the application. After successful login, the user selects the operating mode, either Auto Mode or Manual Mode, based on the simulation requirements.

In the selected mode, the user configures emitter parameters such as frequency type (fixed or agile), PRI type (fixed, jittered, or staggered), pulse width, direction of arrival, and tolerances. Once configuration is completed, the system initiates continuous PDW generation in 2-second time windows, simulating real-time radar emitter behavior.

The generated PDWs from multiple emitters are then interleaved into a single PDW stream, replicating real Electronic Support Measures (ESM) environments. This interleaved PDW dataset is stored and forwarded to the de-interleaving phase.

During de-interleaving, the user selects relevant PDW features such as frequency, PRI, pulse width, and DOA. Density-based clustering algorithms (DBSCAN or HDBSCAN) are applied to segregate the interleaved PDWs into distinct emitter clusters. Each detected cluster is assigned a unique emitter identifier.

Finally, the system displays the results in a structured manner, showing the interleaved PDWs, de-interleaved emitters, and emitter-wise pulse tracking. The user can choose to continue the simulation for subsequent time windows or terminate the process.

Figure 5.3: Flowchart of Radar PDW De-Interleaving Process

5.4 Module Description

The proposed system is divided into multiple functional modules. Each module is designed to perform a specific task in the overall PDW simulation and de-interleaving workflow. The modular approach ensures clarity, scalability, and ease of maintenance while supporting real-time processing requirements.

5.4.1 User Authentication Module

Purpose:
The User Authentication Module ensures secure access to the application by validating authorized users before allowing interaction with the system.

Inputs:

- Username / Email ID

- Password

Processing:
This module verifies user credentials against a secured user database. Passwords are stored in encrypted form to prevent unauthorized access. Based on the user role (admin or user), appropriate access permissions are granted.

Outputs:

- Successful login session

- Access to system dashboard based on user role

5.4.2 Auto Mode Simulation Module

Purpose:
The Auto Mode Simulation Module automatically generates realistic PDW data by simulating multiple radar emitters operating simultaneously.

Inputs:

- Number of emitters

- Emitter type distribution (fixed, agile, staggered PRI)

- Parameter ranges (frequency, PRI, pulse width, DOA, amplitude)

- User-defined tolerance values

Processing:
For each emitter, base parameters are generated within the specified ranges. Controlled variations are applied using tolerance values to simulate real-world radar behavior. PDWs are generated continuously in fixed 2-second time windows and interleaved to represent multiple emitters operating concurrently.

Outputs:

- Interleaved PDW dataset

- Time-of-arrival information in continuous time format

- PDW data stored for further processing

5.4.3 Manual Mode Simulation Module

Purpose:
The Manual Mode Simulation Module allows users to configure radar emitters explicitly to study specific scenarios and validate de-interleaving performance.

Inputs:

- User-defined emitter parameters

- Frequency type (fixed or agile with multiple modes)

- PRI type (fixed, jittered, staggered)

- Pulse width, amplitude, and DOA values

Processing:
Based on the user configuration, PDWs are generated with realistic parameter variations. Continuous time progression is maintained, and PDWs are interleaved across emitters to simulate operational conditions.

Outputs:

- Custom interleaved PDW dataset

- Stored PDWs for real-time de-interleaving analysis

5.4.4 PDW Interleaving Module

Purpose:
The PDW Interleaving Module combines pulses from multiple emitters into a single time-ordered PDW stream, simulating real Electronic Support Measure (ESM) receiver input.

Inputs:

- PDWs generated from Auto Mode or Manual Mode

- Time-of-arrival values

Processing:
PDWs from all emitters are sorted based on their time-of-arrival. This results in a single interleaved stream where pulses from different emitters are mixed in time, closely resembling real-world intercepted radar signals.

Outputs:

- Interleaved PDW stream

- Input dataset for the de-interleaving phase

5.4.5 De-Interleaving Module

Purpose:
The De-Interleaving Module separates interleaved PDWs into individual emitter tracks using density-based clustering algorithms.

Inputs:

- Interleaved PDW dataset

- Selected features (frequency, PRI, pulse width, DOA)

Processing:
Feature scaling is applied to normalize PDW parameters. Density-based clustering algorithms such as DBSCAN and HDBSCAN are used to group pulses belonging to the same emitter based on similarity and density. Algorithm parameters are automatically tuned to achieve optimal clustering performance.

Outputs:

- De-interleaved PDW dataset with emitter identifiers

- Summary of detected emitters and pulse distribution

5.4.6 Emitter Tracking and Analysis Module

Purpose:
This module provides detailed tracking and analysis of individual emitters after de-interleaving.

Inputs:

- De-interleaved PDW dataset

- Selected emitter ID

Processing:
PDWs corresponding to a selected emitter are filtered and displayed. Temporal and parametric characteristics of the emitter are analyzed to understand its operational behavior.

Outputs:

- Emitter-wise PDW details

- Pulse statistics for each detected emitter

5.4.7 Data Storage and Export Module

Purpose:
The Data Storage and Export Module ensures persistence of generated and processed PDW data for offline analysis and reporting.

Inputs:

- Interleaved PDW data

- De-interleaved PDW results

Processing:
PDW datasets are formatted, rounded to appropriate precision, and stored securely in user-specific directories.

Outputs:

- CSV files containing interleaved and de-interleaved PDWs

- Exportable datasets for further research and validation

5.5 Data Flow Diagram

The Data Flow Diagram (DFD) illustrates the logical flow of data within the proposed PDW Simulation and De-Interleaving System. It represents how Pulse Descriptor Word (PDW) data is generated, processed, stored, and analyzed across different functional modules. The DFD focuses on data movement and transformation rather than implementation details, making it suitable for understanding system functionality at an abstract level.

The proposed system processes PDW data in a continuous real-time manner. User-defined parameters are provided as inputs to the system, which then simulates PDWs, interleaves them, performs de-interleaving using clustering algorithms, and finally presents emitter-wise analysis results to the user.

5.5.1 Level-0 Data Flow Diagram

The Level-0 Data Flow Diagram represents the entire PDW Simulation and De-Interleaving System as a single process. It defines the system boundary and shows interaction with external entities.

The primary external entity is the User (Analyst/Operator) who provides configuration parameters such as simulation mode, emitter characteristics, and de-interleaving options. The system processes these inputs and generates outputs in the form of interleaved PDW data, de-interleaved emitter data, and analytical results.

This level provides a high-level understanding of how the system communicates with the user without exposing internal processing details.

Figure 5.5.1: Level-0 Data Flow Diagram of the PDW Simulation and De-Interleaving System

5.5.2 Level-1 Data Flow Diagram

The Level-1 Data Flow Diagram decomposes the system into major functional processes and illustrates the flow of PDW data between them.

The system begins with User Authentication, where user credentials are validated. Once authenticated, the user selects either Auto Mode or Manual Mode for PDW simulation. In Auto Mode, emitter parameters are generated automatically within defined tolerances, whereas in Manual Mode, parameters are explicitly provided by the user.

The generated PDWs are passed to the PDW Interleaving Module, where pulses from multiple emitters are combined into a single interleaved data stream. This interleaved PDW data is temporarily stored in a PDW Buffer.

The De-Interleaving Engine processes the interleaved PDWs using density-based clustering algorithms such as DBSCAN or HDBSCAN. Based on selected features like frequency, PRI, pulse width, and DOA, pulses are grouped emitter-wise.

The de-interleaved output is forwarded to the Emitter Tracking and Analysis Module, which provides emitter-wise pulse statistics and tracking information. The final results are stored in structured output files and displayed to the user for further analysis.

Figure 5.5.2: Level-1 Data Flow Diagram of the PDW Simulation and De-Interleaving System

5.5.3 Data Stores Used

The system utilizes the following data stores:

- User Database – Stores user credentials and access roles.

- PDW Buffer – Temporarily holds continuously generated interleaved PDWs.

- Interleaved PDW File – Stores raw interleaved PDW data.

- De-Interleaved PDW File – Stores emitter-wise separated PDW data after processing.

- IMPLEMENTATION

6.1 Step by Step working of the system

This section explains the complete operational workflow of the proposed PDW Simulation and De-Interleaving System. The system is designed to function as a continuous real-time application, simulating radar pulse environments and performing emitter separation using density-based clustering algorithms.

The working of the system is divided into sequential stages as described below.

Step 1: User Authentication and Access Control

The system begins with a secure login mechanism where users authenticate themselves using valid credentials. Role-based access control is implemented to differentiate between administrative users and standard users. Upon successful authentication, the user gains access to the system dashboard, from which simulation and analysis modules can be accessed.

Step 2: Mode Selection (Auto Mode / Manual Mode)

After login, the user selects the operational mode:

- Auto Mode: The system automatically generates emitter parameters within realistic ranges and tolerances.

- Manual Mode: The user manually configures emitter parameters such as frequency type, PRI type, pulse width, amplitude, and direction of arrival.

This mode selection determines how PDW data is generated in subsequent steps.

Step 3: Configuration of Simulation Parameters

In the selected mode, the user configures simulation parameters including:

- Number of emitters

- Pulses per emitter per time window

- Frequency characteristics (fixed or agile)

- PRI characteristics (fixed, jittered, or staggered)

- Pulse width, amplitude, and DOA

- Tolerance values for frequency, PRI, pulse width, amplitude, and DOA

These configurations ensure that the generated PDWs closely resemble real-world radar signals.

Step 4: Continuous PDW Generation

The system generates PDWs in continuous 2-second time windows, maintaining a global time reference. For each emitter:

- Pulses are generated based on configured parameters.

- Controlled randomness (tolerances) is applied to simulate realistic variations.

- Time of Arrival (TOA) values are incremented sequentially and mapped to real-time format.

PDWs from all emitters are combined into a single interleaved stream, simulating a dense radar signal environment.

Step 5: Interleaved PDW Storage

The interleaved PDW stream is temporarily stored in an in-memory buffer and simultaneously written to a structured output file. This buffer supports continuous data accumulation and enables real-time analysis without interrupting simulation.

Step 6: Feature Selection for De-Interleaving

Before de-interleaving, the user selects relevant PDW features such as:

- Frequency

- Pulse Repetition Interval (PRI)

- Pulse Width (PW)

- Direction of Arrival (DOA)

Feature selection directly influences clustering performance and allows adaptability to different radar environments.

Step 7: De-Interleaving Using Clustering Algorithms

The de-interleaving process uses density-based clustering techniques:

- DBSCAN for fixed-density emitter environments

- HDBSCAN for variable-density and complex signal scenarios

The system automatically tunes algorithm parameters to achieve optimal emitter separation. Pulses belonging to the same radar emitter are grouped into distinct clusters.

Step 8: Emitter Identification and Tracking

Each detected cluster is assigned a unique emitter identifier. The system computes emitter-wise statistics such as:

- Number of pulses

- Mean frequency

- Mean PRI

Users can select individual emitters to view detailed pulse-level tracking information.

Step 9: Visualization and Result Presentation

The system presents results using a structured three-window interface:

- Interleaved PDW data (raw input)

- De-interleaved emitter summary

- Emitter-wise tracked pulse data

This layout ensures clear understanding of how pulses are separated and tracked over time.

Step 10: Data Storage and Export

Finally, both interleaved and de-interleaved PDW datasets are saved in structured CSV format. These files can be used for offline analysis, validation, or further research.

6.2 Algorithms Used

The PDW simulation algorithm is responsible for generating realistic radar pulse data under controlled conditions. It models the behavior of different radar emitters by producing Pulse Descriptor Words (PDWs) based on configurable parameters.

Algorithm Description

- Initialize a global continuous time reference.

- For each emitter:

  - Assign base parameters such as frequency, PRI, pulse width, amplitude, and DOA.

  - Apply predefined tolerances to simulate real-world variations.

- Generate pulses for each emitter within a fixed time window (2 seconds).

- Increment Time of Arrival (TOA) sequentially based on PRI.

- Combine pulses from all emitters into a single interleaved PDW stream.

Emitter Modes Supported

- Fixed frequency emitters

- Frequency agile emitters (multi-mode frequency hopping)

- Fixed PRI emitters

- Jittered PRI emitters

- Staggered PRI emitters

This algorithm ensures that pulses belonging to the same emitter remain statistically consistent while still exhibiting realistic variability.

6.2.1 Density-Based Spatial Clustering of Applications with Noise (DBSCAN)

DBSCAN is a density-based clustering algorithm used to de-interleave PDWs by grouping pulses that exhibit similar characteristics.

Algorithm Overview

DBSCAN clusters data points based on the density of points in feature space, without requiring prior knowledge of the number of clusters.

Key Parameters

- ε (Epsilon): Maximum distance between two points to be considered neighbors.

- MinSamples: Minimum number of points required to form a cluster.

Application in PDW De-Interleaving

- Selected PDW features (frequency, PRI, PW, DOA) are standardized.

- DBSCAN groups pulses that fall within defined tolerance ranges.

- Each cluster corresponds to a unique radar emitter.

- Pulses that do not satisfy density conditions are marked as unassigned.

Advantages

- No need to specify the number of emitters in advance.

- Robust against noise and overlapping signals.

- Well-suited for fixed-density radar environments.

6.2.2 Hierarchical Density-Based Spatial Clustering (HDBSCAN)

HDBSCAN is an extension of DBSCAN that supports variable-density clustering, making it suitable for complex radar environments.

Algorithm Overview

HDBSCAN builds a hierarchical clustering structure and extracts the most stable clusters based on density persistence.

Key Parameters

- Minimum Cluster Size: Minimum number of pulses required to form an emitter cluster.

- Minimum Samples: Controls the conservativeness of clustering.

Application in the System

- PDW feature space is standardized.

- HDBSCAN automatically adapts to variations in pulse density.

- Stable clusters are extracted as radar emitters.

- Less stable pulses remain unassigned.

Advantages

- Handles emitters with different pulse rates.

- Automatically adapts to density variations.

- More robust for mixed or agile emitter environments.

6.2.3 Feature Scaling and Normalization

Before applying clustering algorithms, feature normalization is performed using standard scaling.

Purpose

- Ensures all features contribute equally to distance calculations.

- Prevents domination of clustering results by high-magnitude parameters such as frequency.

Method Used

- StandardScaler (zero mean, unit variance)

6.3 Pseudocode

This section presents the pseudocode for the core modules of the proposed system. Pseudocode is used to describe the logical flow of the algorithms in a language-independent manner, making the system behavior easy to understand without requiring knowledge of implementation-specific details.

6.3.1 Pseudocode for PDW Simulation Module

Purpose:
To generate realistic and interleaved Pulse Descriptor Words (PDWs) for multiple radar emitters under continuous-time operation.

Code:

BEGIN PDW_SIMULATION

Initialize global_time ← 0

Initialize PDW_Buffer ← empty

FOR each 2-second time window DO

window_start ← global_time

window_end ← global_time + 2 seconds

FOR each emitter i = 1 to Number_of_Emitters DO

Assign base_frequency[i]

Assign base_PRI[i]

Assign base_pulse_width[i]

Assign base_amplitude[i]

Assign base_DOA[i]

Set frequency_modes based on emitter type

Set PRI_modes based on PRI type

Apply predefined tolerances to all parameters

Generate initial TOA randomly within window

FOR pulse = 1 to Pulses_Per_Emitter DO

Select frequency from frequency_modes

Select PRI from PRI_modes

Add random tolerance to frequency, PRI, PW, DOA, amplitude

Create PDW:

PDW.frequency ← frequency

PDW.PRI ← PRI

PDW.PW ← pulse_width

PDW.DOA ← DOA

PDW.Amplitude ← amplitude

PDW.TOA ← current TOA

Append PDW to PDW_Buffer

Update TOA ← TOA + PRI

IF TOA > window_end THEN

BREAK

END IF

END FOR

END FOR

Update global_time ← window_end

END FOR

END PDW_SIMULATION

6.3.2 Pseudocode for Feature Selection Module

Purpose:
To select relevant PDW parameters for de-interleaving based on user choice.

Code:

BEGIN FEATURE_SELECTION

Initialize Selected_Features ← empty list

IF Frequency selected THEN

Add frequency to Selected_Features

END IF

IF PRI selected THEN

Add PRI to Selected_Features

END IF

IF Pulse Width selected THEN

Add PW to Selected_Features

END IF

IF DOA selected THEN

Add DOA to Selected_Features

END IF

Return Selected_Features

END FEATURE_SELECTION

6.3.3 Pseudocode for De-Interleaving using DBSCAN

Purpose:
To separate interleaved PDWs into distinct radar emitters using density-based clustering.

Code:

BEGIN DBSCAN_DEINTERLEAVING

Input: PDW_Data, Selected_Features

Normalize PDW_Data using standard scaling

IF ground_truth_emitters available THEN

Auto-tune epsilon value

END IF

Apply DBSCAN clustering:

labels ← DBSCAN(PDW_Data, epsilon, min_samples)

FOR each label in labels DO

IF label == noise THEN

Assign Emitter_ID ← 0

ELSE

Assign unique Emitter_ID

END IF

END FOR

Return clustered PDWs with Emitter_IDs

END DBSCAN_DEINTERLEAVING

6.3.4 Pseudocode for De-Interleaving using HDBSCAN

Purpose:
To perform adaptive de-interleaving in variable-density radar environments.

Code:

BEGIN HDBSCAN_DEINTERLEAVING

Input: PDW_Data, Selected_Features

Normalize PDW_Data

Automatically determine stable cluster sizes

Apply HDBSCAN clustering:

labels ← HDBSCAN(PDW_Data)

FOR each detected cluster DO

Assign unique Emitter_ID

END FOR

Mark unstable pulses as unassigned

Return clustered PDWs with Emitter_IDs

END HDBSCAN_DEINTERLEAVING

6.3.5 Pseudocode for Emitter Tracking

Purpose:
To enable pulse-wise tracking of each detected emitter.

Code:

BEGIN EMITTER_TRACKING

Input: Deinterleaved_PDWs

FOR each unique Emitter_ID DO

Extract pulses belonging to Emitter_ID

Sort pulses by TOA

Display pulse parameters for analysis

END FOR

END EMITTER_TRACKING

- RESULTS

7.1 Output Screenshots

This subsection presents the output screens generated at various stages of the proposed PDW de-interleaving system. The screenshots demonstrate the correctness, usability, and effectiveness of the implemented solution under different operating scenarios.

Each screenshot corresponds to a major functional stage of the system, starting from PDW simulation to final emitter tracking.

7.1.1 Auto Mode – PDW Simulation Output

The above screenshot shows the Auto Mode PDW simulation module. Multiple radar emitters with fixed, agile, and staggered PRI characteristics are simulated over continuous 2-second time windows. The generated PDWs include realistic variations in frequency, PRI, pulse width, amplitude, and direction of arrival (DOA). The output confirms successful generation of interleaved PDW streams in a real-time simulation environment.

7.1.2 Manual Mode – Configured Emitter Simulation Output

This screenshot illustrates the Manual Mode of PDW simulation, where the user explicitly configures individual emitter characteristics such as frequency type (fixed/agile), PRI type (fixed, jittered, staggered), pulse width, amplitude, and DOA. The output demonstrates that the system accurately generates PDWs based on user-defined configurations while maintaining continuous time progression.

7.1.3 Interleaved PDW Data View

The above screenshot shows the interleaved PDW data obtained from the simulation phase. Pulses from multiple emitters are mixed together in time, closely representing real Electronic Support Measure (ESM) environments. This interleaved dataset serves as the input to the de-interleaving algorithms.

7.1.4 De-Interleaved Emitter Summary

This screenshot presents the de-interleaving results after applying density-based clustering algorithms. Each detected emitter is assigned a unique identifier, along with its corresponding pulse count and average parameter values. The output confirms that the system successfully separates interleaved pulses into distinct radar emitters.

7.1.5 Emitter Tracking Output

The above screenshot shows the emitter tracking view. Upon selecting an emitter ID, all pulses associated with that emitter are displayed in chronological order. This enables detailed analysis of emitter behavior, validating the correctness of the de-interleaving process and supporting further signal intelligence tasks.

7.1.6 De-Interleaving Summary Statistics

This screenshot summarizes the overall de-interleaving performance. The system reports the number of detected emitters, compares it with the expected count, and indicates the number of unassigned pulses. Minor deviations occur due to overlapping parameter spaces and density variations, which are inherent in realistic radar environments.

7.2 Observations

The following observations were recorded after executing the proposed system under multiple simulation scenarios using both Auto Mode and Manual Mode. The behavior of the de-interleaving algorithms was analyzed by varying emitter configurations, feature selections, and clustering parameters.

7.2.1 Effectiveness of PDW Simulation

- The Auto Mode successfully generated realistic interleaved PDW streams with fixed, frequency-agile, and staggered PRI emitters operating simultaneously.

- Continuous time-based generation (2-second windows) closely replicated real-time Electronic Support Measure (ESM) environments.

- Manual Mode allowed precise control over emitter parameters, enabling the creation of controlled test scenarios for algorithm validation.

Observation:
The quality and realism of the simulated PDWs played a critical role in achieving accurate de-interleaving results.

7.2.2 Impact of Feature Selection on Clustering

- Using frequency and PRI as primary features resulted in stable and accurate clustering when emitters were well-separated in parameter space.

- Including additional features such as pulse width (PW) and DOA improved clustering performance when emitter parameters overlapped.

- However, adding multiple features increased data dimensionality, sometimes leading to cluster fragmentation if tolerances were not properly controlled.

Observation:
Careful feature selection is essential; unnecessary features may degrade clustering accuracy in high-density scenarios.

7.2.3 DBSCAN Performance Characteristics

- DBSCAN effectively identified clusters corresponding to individual emitters without prior knowledge of the number of emitters.

- The auto-tuning mechanism for the eps parameter significantly reduced manual intervention and improved consistency across datasets.

- Small variations in emitter tolerances occasionally resulted in a few pulses being marked as unassigned.

Observation:
DBSCAN performs reliably when emitter tolerances are realistic and density variations are controlled.

7.2.4 HDBSCAN Performance Characteristics

- HDBSCAN demonstrated better adaptability in datasets with varying pulse densities and overlapping emitter parameters.

- The number of detected clusters varied with changes in minimum cluster size, highlighting its sensitivity to density hierarchy.

- Automatic cluster selection provided reasonable results, but fine-grained control improved accuracy in complex scenarios.

Observation:
HDBSCAN is more suitable for heterogeneous environments but requires careful interpretation of its output.

7.2.5 Interpretation of Unassigned Pulses

- A small number of pulses were occasionally marked as unassigned during de-interleaving.

- These pulses typically occurred at:

  - Boundaries between overlapping emitter parameter ranges

  - Transition regions of agile frequency or staggered PRI patterns

Observation:
Unassigned pulses do not necessarily indicate noise; they represent ambiguous pulses that cannot be confidently associated with a single emitter under density-based constraints.

7.2.6 Validation Against Ground Truth

- In controlled scenarios with known emitter counts, the system consistently detected emitter numbers close to the ground truth.

- Minor deviations were observed in high-density or highly agile scenarios, reflecting realistic operational challenges.

Observation:
The de-interleaving results closely matched expected outcomes, validating the correctness of the simulation and clustering approach.

7.2.7 Overall System Behavior

- The integration of simulation, de-interleaving, and emitter tracking provided a clear end-to-end workflow.

- The three-window display significantly improved interpretability and user confidence in algorithm performance.

- The system remained stable under increased emitter counts and pulse densities.

Observation:
The proposed system demonstrates robustness, scalability, and practical applicability for real-time PDW de-interleaving tasks.

`

7.3 Performance Analysis

This section evaluates the performance of the proposed PDW de-interleaving system under various simulation scenarios. The analysis focuses on clustering accuracy, robustness, scalability, and algorithm behavior under increasing system complexity.

7.3.1 De-Interleaving Accuracy

- The system achieved high de-interleaving accuracy when emitter parameters were well-separated and realistic tolerances were maintained.

- For fixed-frequency and fixed-PRI emitters, both DBSCAN and HDBSCAN correctly identified individual emitters with minimal misclassification.

- In scenarios involving frequency-agile and staggered PRI emitters, the system maintained stable clustering with only a small number of ambiguous pulses.

Performance Insight:
Accurate simulation with controlled tolerances directly improves clustering reliability.

7.3.2 Impact of Emitter Density

- As the number of emitters increased, the density of PDWs in the feature space also increased.

- DBSCAN performed consistently up to moderate emitter densities, after which minor cluster fragmentation was observed.

- HDBSCAN showed better adaptability in high-density environments due to its hierarchical clustering approach.

Performance Insight:
Emitter density is a critical factor influencing clustering performance, especially in dense operational environments.

7.3.3 Feature Dimensionality Analysis

- Clustering using two features (Frequency, PRI) resulted in faster execution and stable clusters in simple scenarios.

- Adding Pulse Width and DOA improved discrimination in overlapping scenarios but increased computational complexity.

- Excessive feature inclusion without proper normalization led to over-segmentation.

Performance Insight:
An optimal balance between feature richness and dimensionality is necessary for stable performance.

7.3.4 Algorithm Scalability

- The system successfully handled increasing numbers of pulses per emitter without significant degradation in performance.

- Continuous 2-second PDW generation windows enabled incremental processing without memory overload.

- The clustering algorithms scaled linearly with the number of PDWs for most test cases.

Performance Insight:
The system is scalable and suitable for extended monitoring durations.

7.3.5 Robustness to Parameter Variations

- Minor variations in frequency, PRI, pulse width, and DOA (within defined tolerances) did not significantly affect cluster integrity.

- Larger deviations increased the number of unassigned pulses, reflecting realistic uncertainty in signal measurements.

Performance Insight:
The system remains robust under realistic signal fluctuations.

7.3.6 DBSCAN vs HDBSCAN Comparison

| Criterion | DBSCAN | HDBSCAN |
| --- | --- | --- |
| Need for Cluster Count | Not Required | Not Required |
| Sensitivity to Density | Moderate | Low |
| Performance in Dense Scenarios | Moderate | High |
| Stability Across Scenarios | High (with tuning) | High |
| Interpretability | High | Moderate |

Performance Insight:
DBSCAN is well-suited for structured environments, while HDBSCAN performs better in complex and heterogeneous scenarios.

7.3.7 Real-Time Applicability

- The continuous PDW generation and step-wise de-interleaving closely resemble real-time ESM operation.

- The system demonstrated consistent response times suitable for near real-time analysis.

Performance Insight:
The proposed solution is suitable for operational Electronic Support Measure systems with real-time constraints.

- TESTING

The testing phase was carried out to validate the correctness, reliability, and robustness of the proposed PDW de-interleaving system. Multiple test scenarios were designed to ensure that the system meets all functional requirements and performs accurately under varying operational conditions.

Testing was conducted using simulated PDW data generated through both Auto Mode and Manual Mode, representing realistic Electronic Support Measure (ESM) environments.

8.1 Test Cases

The following test cases were designed to verify individual functionalities of the system as well as end-to-end system behavior.

Test Case 1: Fixed Emitter De-Interleaving

- Description: Validate de-interleaving of emitters with fixed frequency and fixed PRI.

- Input: PDWs generated with constant frequency and PRI values.

- Expected Result: Each emitter forms a distinct cluster.

- Actual Result: Emitters were correctly identified with stable clusters.

- Status: Pass

Test Case 2: Frequency Agile Emitter De-Interleaving

- Description: Test de-interleaving of frequency-agile emitters with multiple frequency modes.

- Input: PDWs generated with multiple frequency values per emitter.

- Expected Result: Pulses belonging to the same agile emitter are grouped into one cluster.

- Actual Result: Agile emitters were correctly clustered.

- Status: Pass

Test Case 3: Staggered PRI Emitter De-Interleaving

- Description: Validate handling of emitters with staggered PRI patterns.

- Input: PDWs with alternating PRI values.

- Expected Result: Pulses are grouped based on emitter behavior despite PRI variation.

- Actual Result: Staggered PRI emitters were correctly identified.

- Status: Pass

Test Case 4: Multi-Emitter Mixed Scenario

- Description: Test system with fixed, agile, and staggered emitters operating simultaneously.

- Input: Mixed PDW stream with multiple emitter types.

- Expected Result: Correct identification of all emitters.

- Actual Result: All emitters were detected with minimal unassigned pulses.

- Status: Pass

Test Case 5: Feature Combination Testing

- Description: Evaluate clustering performance using different feature combinations.

- Input: PDWs clustered using (Frequency, PRI), and (Frequency, PRI, PW, DOA).

- Expected Result: Improved separation with additional features.

- Actual Result: Feature-rich clustering improved discrimination but required careful tuning.

- Status: Pass

Test Case 6: High Emitter Density Scenario

- Description: Test system performance with a large number of emitters.

- Input: Increased emitter count and pulse density.

- Expected Result: System maintains clustering accuracy.

- Actual Result: Minor fragmentation observed at extreme densities.

- Status: Pass (with limitations)

8.2 Test Results

- All primary functional requirements were successfully validated.

- The system consistently produced accurate de-interleaving results under normal and moderate load conditions.

- Minor deviations were observed in extremely dense scenarios, which are expected in real-world ESM systems.

- CONCLUSION

9.1 Summary of work done

This project successfully designed and implemented a comprehensive Pulse Descriptor Word (PDW) de-interleaving system tailored for Electronic Support Measures (ESM) applications. The system simulates realistic radar environments by generating continuous PDW streams with configurable emitter characteristics, including fixed, frequency-agile, and staggered PRI emitters. A real-time simulation framework was developed to model practical operational conditions encountered in electronic warfare scenarios.

The core functionality of the system focuses on accurate de-interleaving of interleaved PDWs using density-based clustering techniques. DBSCAN and HDBSCAN algorithms were implemented and evaluated for their ability to identify and separate multiple emitters without prior knowledge of emitter count. Feature-based clustering using frequency, PRI, pulse width, and direction of arrival enabled effective discrimination of emitters under varying signal behaviors and tolerances.

An interactive user interface was developed to support both automatic and manual simulation modes, allowing users to configure emitter parameters, visualize interleaved and de-interleaved data, and track individual emitters in detail. The system was validated through extensive testing across multiple scenarios, demonstrating reliable performance, adaptability, and robustness. Overall, the project delivers a practical, modular, and scalable solution for PDW de-interleaving aligned with real-world ESM requirements.

9.2 Achievements

- Successfully designed and developed a real-time PDW simulation framework capable of generating continuous interleaved pulse streams for multiple radar emitters with realistic operational behavior.

- Implemented an effective PDW de-interleaving mechanism using density-based clustering algorithms (DBSCAN and HDBSCAN) to accurately identify and separate emitters without prior knowledge of their count.

- Achieved robust emitter discrimination by incorporating multi-parameter feature analysis, including frequency, PRI, pulse width, and direction of arrival, improving clustering reliability under varying scenarios.

- Developed a user-friendly interactive application supporting both automatic and manual configuration modes, enabling flexible experimentation and validation of de-interleaving performance.

- Enabled emitter-wise tracking and analysis, providing clear visualization of interleaved data, de-interleaved results, and individual emitter pulse characteristics for validation and assessment.

- Validated system performance across multiple test scenarios, demonstrating consistency, scalability, and adaptability to different emitter configurations and operational conditions.

9.3 Learning Outcomes

- Gained a strong understanding of Radar Electronic Support Measures (ESM) and the role of PDW analysis in emitter identification and situational awareness.

- Acquired practical experience in PDW simulation and real-time data modeling, including the generation of fixed, agile, and staggered PRI emitter behaviors.

- Developed hands-on expertise in density-based clustering algorithms such as DBSCAN and HDBSCAN for solving real-world signal de-interleaving problems.

- Learned the importance of feature selection, tolerance modeling, and parameter normalization in improving clustering accuracy and system robustness.

- Enhanced skills in system design and software development, including modular architecture, interactive UI design, and real-time data processing.

- Improved ability to analyze algorithm performance, interpret clustering outcomes, and refine system behavior based on experimental observations.

- Strengthened documentation and reporting skills by preparing a structured SDLC-based technical report aligned with defense research standards.

- FUTURE SCOPE

10.1 Possible Enhancements

- Incorporation of Noise and Outlier Modeling:
Future versions of the system can include realistic noise pulses and outlier PDWs to simulate complex operational environments and evaluate algorithm robustness under degraded conditions.

- Adaptive and Hybrid De-Interleaving Algorithms:
The system can be extended to use hybrid approaches combining DBSCAN, HDBSCAN, and rule-based logic to automatically adapt to varying emitter densities and signal characteristics.

- Advanced Feature Engineering:
Additional PDW features such as pulse train patterns, frequency hopping sequences, and higher-order PRI statistics can be integrated to further improve emitter discrimination accuracy.

- Online and Incremental Clustering:
Implementing true online clustering techniques would allow continuous de-interleaving without reprocessing historical data, enabling closer alignment with real-time ESM systems.

- Scalability for Large-Scale Scenarios:
The framework can be optimized for handling thousands of emitters and high-rate pulse streams using parallel processing or GPU acceleration.

- Integration with Threat Libraries:
De-interleaved emitters can be matched against predefined radar threat libraries to enable automatic emitter classification and threat assessment.

- Enhanced Visualization and Analytics:
Future enhancements may include advanced dashboards, timeline-based visualizations, and statistical confidence indicators for tracked emitters.

REFERENCES

- Skolnik, M. I., Introduction to Radar Systems, 3rd Edition, McGraw-Hill Education, 2001.

- Wiley, R. G., Electronic Intelligence: The Analysis of Radar Signals, Artech House, 2006.

- Adamy, D. L., EW 101: A First Course in Electronic Warfare, Artech House, 2001.

- Schleher, D. C., Electronic Warfare in the Information Age, Artech House, 1999.

- Nandi, A. K., & Azzouz, E. E., “Algorithms for Automatic Modulation Recognition of Communication Signals,” IEEE Transactions on Communications, vol. 46, no. 4, pp. 431–436, 1998.

- Xu, R., & Wunsch, D., Clustering, Wiley-IEEE Press, 2009.

- Ester, M., Kriegel, H.-P., Sander, J., & Xu, X.,
“A Density-Based Algorithm for Discovering Clusters in Large Spatial Databases with Noise,”
Proceedings of the 2nd International Conference on Knowledge Discovery and Data Mining (KDD), 1996.

- Campello, R. J. G. B., Moulavi, D., & Sander, J.,
“Density-Based Clustering Based on Hierarchical Density Estimates,”
Advances in Knowledge Discovery and Data Mining, Springer, 2013.

- Mahalanobis, P. C., “On the Generalized Distance in Statistics,” Proceedings of the National Institute of Sciences of India, vol. 2, pp. 49–55, 1936.

- Richards, M. A., Fundamentals of Radar Signal Processing, 2nd Edition, McGraw-Hill Education, 2014.

- IEEE Standard Dictionary of Electrical and Electronics Terms, IEEE Press.

- Scikit-Learn Developers,
Scikit-Learn: Machine Learning in Python,
https://scikit-learn.org

- McInnes, L., Healy, J., & Astels, S.,
“hdbscan: Hierarchical Density Based Clustering,”
Journal of Open Source Software, 2017.